ECA 1212 Test 1

1.
Using mesh current analysis, find the currents i1 and i2 for the circuit below:

[image: image1]
[10 marks] (CO2)

Solution:

Loop 1:

1 = 1(i1) + 3(i1−i2)

1 = 4i1 − 3i2
3 = 12i1 − 9i2


(1)

Loop 2:

−2 = 3(i2−i1) + 2(i2)

−2 = −3i1 + 5i2
−8 = −12i1 + 20i2


(2)

(1) + (2)
−5 = 11i2
i2 = 
[image: image2.wmf]11

5

-

A
Then, 
1 = 4i1 − 3
[image: image3.wmf]÷

ø

ö

ç

è

æ

-

11

5


1 = 4i1 + 
[image: image4.wmf]11

15



[image: image5.wmf]11

15

11

-

 = 4i1
i1 = 
[image: image6.wmf]44

4

-

 = 
[image: image7.wmf]11

1

-

A
2.
Find the Thévenin’s equivalent of the circuit below for the part on the left of RL:


[image: image8]
[10 marks] (CO2)

Solution:

RTh
= (((8 || 8) + 2) || 3) + 8

= ((4 + 2) || 3) + 8

= (6 || 3) + 8


= 2 + 8 = 10Ω

[image: image9]
The equivalent resistance from the 3 bottom resistors is:

8 || (2 + 3)
= 8 || 5


= 
[image: image10.wmf]÷

ø

ö

ç

è

æ

+

8

5

40



= 3.08Ω

Hence by voltage divider rule, the voltage between point c and b is:
= 12 × 
[image: image11.wmf]÷

ø

ö

ç

è

æ

+

08

.

3

8

08

.

3

 = 3.34V

Voltage between point a and b:
= 3.34 × 
[image: image12.wmf]÷

ø

ö

ç

è

æ

+

2

3

3

 = 2V# = VTh
3.
Assuming Vγ = 0.75V, determine the DC configuration (VCE, IC and IB) of the circuit below:


[image: image13]
[10 marks] (CO1)

Solution:

VBB = 
[image: image14.wmf]÷

ø

ö

ç

è

æ

+

16

10

10

20

 = 7.69V
RB = 16 || 10 = 6.15Ω
KVL at input:
	7.69
	=
	6.15IB + 0.75 + 3k(IB+IC)

	7.69
	=
	6.15IB + 0.75 + 3k(IB+βIB)

	6.94
	=
	6.15IB + 3kIB(1+70)

	6.94
	=
	6.15IB + 213000IB

	IB
	=
	32.58 μA


IC = βIB = (70)(32.58μ) = 2.28 mA
	VCE
	=
	20 − 3kIC − 3k(IC+IB)

	VCE
	=
	20 − 3k(2.28m) − 3k(2.28m+32.28μ)

	VCE
	=
	20 − 6.84 − 3k(2.31228m)

	VCE
	=
	13.16 − 6.94

	VCE
	=
	6.22 V


4.
Explain what depletion region is and how it is formed at the interface of PN junction.

[5 marks] (CO1)
Solution:

Depletion region is the area at the interface between p-type and n-type in PN junction that is lacking of majority carrier. It is formed due to the migration of electrons from the n-type side into the p-type side. The migrated electrons combined with the holes majority in the p-type. This migration also causes charge imbalance at the interface which induces electric field. The direction of the field is in the opposite direction of the electrons migration. After sufficient migration the strength of the field is enough to counter balance the migration hence preventing further migration and preventing infinitely large depletion area.
5.
Explain the operation of p-channel depletion mode FET with regards to the way it conducts current.
[5 marks] (CO1)
Solution:

p-channel depletion mode FET is the p-channel FET that conducts current when the gate is grounded. Giving voltage to the gate will reduce the current. The voltage at the gate will induce a repelling field in the channel that pushes holes to the side and reducing the effective current. Once the field is strong enough, it stops the current all together.




























































c





b





a





3Ω





8Ω





2Ω





8Ω





12V





1V





1Ω





2Ω





3Ω





i2





2V





i1





12V





8Ω





2Ω





8Ω





RL





3Ω





8Ω














β = 70





3kΩ





3kΩ





10kΩ





16kΩ





20V








_1386441099.unknown

_1386441262.unknown

_1386441432.unknown

_1386666786.unknown

_1386441431.unknown

_1386441173.unknown

_1385715904.unknown

_1386440964.unknown

_1385716097.unknown

_1385715801.unknown

