EEE 1021

Introduction to Electrical and Electronics Engineering

Tutorial 3

Capacitors and Inductors

Legend:

[B] – Boylestad, 10th Edition, Introductory Circuit Analysis

[F] – Floyd, 6th Edition, Principles of Electric Circuits

1.	Find the voltage across and charge on each capacitor for the circuit of Figure 3.1 - [B] Problem 45 page 432

10V

Figure 3.1

6µF

12µF

6µF

(a)

40V

1200pF

400pF

600pF

(b)

200pF

C1

C2

C3

C1

C2

C3

C4

ans: (a): V1 = 10V, Q1 = 60µC, V2 = 6.67V, Q2 = 40µC, V3 = 3.33V, Q3 = 40µC

(b): V1 = 8V, Q1 = 9.6nC, V2 = 16V, Q2 = 3.2nC, V3 = 16V, Q3 = 6.4nC, V4 = 16V, Q4 = 9.6nC

2kΩ

48V

Figure 3.2

0.04µF

0.08µF

(a)

(b)

4kΩ

6kΩ

80V

60µF

4kΩ

5kΩ

40µF

C1

C2

C1

C2

2.	Find the circuits of Figure 3.2, find the voltage across and charge on each capacitor after each capacitor has charge to its final value - [B] Problem 48 page 433

ans: (a): V1 = 48V, Q1 = 1.92µC, V2 = 32V, Q2 = 2.56µC

(b): V1 = 80V, Q1 = 3.2mC, V2 = 48V, Q2 = 2.88mC

3.	Find the total inductance of the circuits of Figure 3.3 - [B] Problem 31 page 517

LT

4H

2H

3H

6H

(a)

LT

6H

4H

(b)

3.6H

12H

Figure 3.3

ans: (a) 8H, (b) 4H